8086 Assembly Directives

- Assembly language consists of two types of statements:

- Executable statements- These are the statements to be executed by the processor. It consists of the entire instruction set of 8086.

- Assembler directives- These are the statements that direct the assembler to do something. As the name says, it directs the assembler to do a task.

- The specialty of these statements is that they are effective only during the assembly of a program but they do not generate any code that is machine executable.

Different Directives

ASSUME: Assume logical segment name.

- The ASSUME directive is used to inform the assembler the names of the logical segments to be assumed for different segments used in the program .

- In the ALP each segment is given name.

Syntax: ASSUME segreg:segname,…segreg:segname

Ex: ASSUME CS:CODE

ASSUME CS:CODE,DS:DATA,SS:STACK

DB: Define Byte

The DB directive is used to reserve byte or bytes of memory locations in the available memory.

Syntax: Name of variable DB initialization value.

Ex: MARKS DB 35H,30H,35H,40H

NAME DB “VARDHAMAN”

DW: Define Word

- The DW directive serves the same as the DB directive, but it reserves the number of memory words(16-bit) instead of bytes.

Syntax: variable name DW initialization values.

Ex: WORDS DW 1234H,4567H,2367H

DD: Define Double:

- The directive DD is used to define a double word (4bytes) variable.

Syntax: variablename DD 12345678H

Ex: Data1 DD 12345678H

DQ: Define Quad Word

- This directive is used to direct the assembler to reserve 4 words (8 bytes) of memory for the specified variable and may initialize it with the specified values.

Syntax: Name of variable DQ initialize values.

Ex: Data1 DQ 123456789ABCDEF2H

DT: Define Ten Bytes

- The DT directive directs the assembler to define the specified variable requiring 10 bytes for its storage and initialize the 10-bytes with the specified values.

Syntax: Name of variable DT initialize values.

Ex: Data1 DT 123456789ABCDEF34567H

END: End of Program

- The END directive marks the end of an ALP.

- The statement after the directive END will be ignored by the assembler.

ENDP: End of Procedure

- The ENDP directive is used to indicate the end of procedure.

- In the AL programming the subroutines are called procedures.

Ex: Procedure Start

:

Start ENDP

ENDS: End of segment

The ENDS directive is used to indicate the end of segment.

Ex: DATA SEGMENT

:

DATA ENDS

EVEN: Align on Even memory address

- The EVEN directives updates the location counter to the next even address.

Ex: EVEN Procedure Start

:

Start ENDP

The above structure shows a procedure START that is to be aligned at an even address.

EQU: Equate

- The directive EQU is used to assign a label with a value or symbol.

Ex: LABEL EQU 0500H

ADDITION EQU ADD

EXTRN: External and public

- The directive EXTRN informs the assembler that the names, procedures and labels declared after this directive have been already defined in some other AL modules.

- While in other module, where names, procedures and labels actually appear, they must be declared public using the PUBLIC directive.

Ex: MODULE1 SEGMENT

PUBLIC FACT FAR

MODULE1 ENDS

MODULE2 SEGMENT

EXTRN FACT FAR

MODULE2 END

GROUP: Group the related segments

- This directive is used to form logical groups of segments with similar purpose or type.

Ex: PROGRAM GROUP CODE, DATA, STACK

*CODE, DATA and STACK segments lie within a 64KB memory segment that is named as PROGRAM.

LABEL: label

- The label is used to assign name to the current content of the location counter.

Ex: CONTINUE LABEL FAR

- The label CONTINUE can be used for a FAR jump, if the program contains the above statement.

LENGTH: Byte length of a label This is used to refer to the length of a data array or a string

Ex : MOV CX, LENGTH ARRAY

LOCAL: The labels, variables, constant or procedures are declared LOCAL in a module are to be used only by the particular module.

Ex : LOCAL a, b, Data1, Array, Routine

NAME: logical name of a module

- The name directive is used to assign a name to an assembly language program module.

- The module may now be refer to by its declared name. Ex : Name “addition”

OFFSET: offset of a label

- When the assembler comes across the OFFSET operator along with a label, it first computing the 16-bit offset address of a particular label and replace the string ‘OFFSET LABEL’ by the computed offset address.

Ex : MOV SI, offset list

ORG: origin

- The ORG directive directs the assembler to start the memory allotment for the particular segment, block or code from the declared address in the ORG statement.

Ex: ORG 1000H

PROC: Procedure

- The PROC directive marks the start of a named procedure in the statement.

Ex: RESULT PROC NEAR

ROUTINE PROC FAR

PTR: pointer

- The PTR operator is used to declare the type of a label, variable or memory operator.

Ex : MOV AL, BYTE PTR [SI]

MOV BX, WORD PTR [2000H]

SEG: segment of a label

- The SEG operator is used to decide the segment address of the label, variable or procedure.

Ex : MOV AX, SEG ARRAY

MOV DS, AX

SEGMENT: logical segment The segment directive marks the starting of a logical segment

Ex: CODE SEGMENT

:

CODE ENDS

SHORT: The SHORT operator indicates to the assembler that only one byte is required to code the displacement for jump.

Ex : JMP SHORT LABEL

TYPE: The TYPE operator directs the assembler to decide the data type of the specified label and replaces the TYPE label by the decided data type.

- For word variable, the data type is 2.

- For double word variable, the data type is 4.

- For byte variable, the data type is 1.

Ex : STRING DW 2345H, 4567H

MOV AX, TYPE STRING

AX=0002H

GLOBAL: The labels, variables, constants or procedures declared GLOBAL may be used by other modules of the program.

Ex : ROUTINE PROC GLOBAL.

FAR PTR: This directive indicates the assembler that the label following FAR PTR is not available within the same segment and the address of the label is of 32-bits i.e 2-bytes of offset followed by 2-bytes of segment address.

Ex : JMP FAR PTR LABEL

NEAR PTR: This directive indicates that the label following NEAR PTR is in the same segment and needs only 16-bit i.e 2-byte offset to address it

Ex : JMP NEAR PTR LABEL

CALL NEAR PTR ROUTINE